Robotics 2020 YetiBorg Racing

Basic Instructions

February 27th 2020

Do not hesitate to contact me (erwin@liacs.nl),

if you encounter any problems getting started!

- To start the YetiBorg insert the 9V battery. The YetiBorg will now boot. Wait until the green-light becomes stable.

- After the YetiBorg booted it will automatically connect to a wireless network (2.4G) with SSID lmlrobotics and password (as given in the Racing Team Document). Note: A wireless network with these characteristics is available just in front of the LIACS Media Lab (right to the reception) and in the Robotics Lab (Room 404).

- Login with a notebook to the same wireless network and use ssh or RealVNC to connect to the Yetiborg. Note the name of the Yetiborg or the mentioned IP-address that most probably will be assigned to the YetiBorg when connecting to the wireless access point at the LIACS Media Lab.

- Once connected to the YetiBorg you can for example add credentials for other wireless networks.

- Always shut down the YetiBorg before detaching the 9V battery. Do this by issuing a command through ssh, or by selecting <shut down> from the upper left menu. (Failing to do so may corrupt the SD-Card. You also may want to make a copy of the SD-Card using the SD Card Copier tool in <Accessories>.)

The following necessary packages already have been installed:

- bash <(curl https://www.piborg.org/installer/install-picoborgrev.txt)

- bash <(curl https://www.piborg.org/installer/install-zeroborg.txt)

- bash <(curl https://www.piborg.org/installer/install-yetiborg-v2.txt)

- sudo apt-get -y install python-picamera

- sudo apt-get -y install libcv-dev libopencv-dev python-opencv

The resolution of your screen can be set by:

<Raspberry Upper Left><Preferences><Raspberry Pi Configuration><System>[Set Resolution]

For further information and code examples please see the related articles on page:

https://www.piborg.org/robots-1/yetiborg-v2

Assignment, due Monday March 2nd 2020 at 14.00:

Check that the camera works by issuing the following command in a terminal on the YetiBorg:

In a terminal issue the command:  raspistill -o cam.jpg

Check that the motors work by issuing the following commands in a terminal on the YetiBorg:

cd /home/pi/yetiborgv2

./yeti2Sequence.py

Send an e-mail of your findings to erwin@liacs.nl  with subject Robotics YetiBorg Findings.